FilMachine Simulator v2

Piano Completo di Validazione Software

ESP32-S3 + LVGL 9.2.2 | Firmware per sviluppo pellicola fotografica

| Metriche | Valore |
| --- | --- |
| Test totali | 109 |
| Superati (automatici) | 35 |
| Da implementare (automatici) | 63 |
| Da eseguire (manuali) | 11 |
| CRITICAL da fare | 15 |

Marzo 2026

# Indice

# 1. Sintesi

Questo documento definisce 109 test di validazione per il firmware FilMachine Simulator v2 (ESP32-S3, LVGL 9.2.2). Dei 109 test, 35 sono già implementati e superati nella suite automatica corrente (106 test). I rimanenti 74 test coprono aree non ancora verificate, identificate durante l'analisi del codice post-refactoring.

Priorità più alta: la macchina a stati del checkup (transizioni processStep 0→4, timer callbacks, scenari di Stop) con 15 test CRITICAL da implementare. Questi coprono il cuore dell'applicazione: l'esecuzione automatica del processo di sviluppo pellicola.

# 2. Legenda

| Simbolo | Significato |
| --- | --- |
| CRITICAL | Blocca il rilascio. Funzionalità core non testata |
| HIGH | Importante. Rischio regressione o memory leak |
| MEDIUM | Utile. Migliora copertura e robustezza |
| LOW | Nice to have. Edge case minori |
| Automatico | Implementabile nella suite test C (test_runner) |
| Manuale | Richiede interazione umana o hardware reale |
| Superato | Test già implementato e superato nella suite corrente |
| Da fare | Test da implementare |

# 3. Test per Categoria

## 3.1 Checkup State Machine

14 test totali — 0 superati, 14 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| CSM-01 | Step 0 initial state | Verify processStep=0, all timers NULL, UI shows tank/chemistry selection, Start enabled | CRITICAL | Automatico | Da fare |
| CSM-02 | Step 0 → 1: Start first step | Click Start at step 0. handleFirstStep() called, processStep becomes 1, pumpTimer created for water fill | CRITICAL | Automatico | Da fare |
| CSM-03 | Step 1: water fill in progress | During step 1, pumpTimer fires repeatedly. Verify fill percentage updates, pump relay activated | CRITICAL | Automatico | Da fare |
| CSM-04 | Step 1 Skip button | Click Skip during fill. pumpTimer deleted, transition to step 2 (temp control) | CRITICAL | Automatico | Da fare |
| CSM-05 | Step 1 → 2: fill complete | pumpTimer completes fill cycle. pumpTimer deleted, tempTimer created, processStep=2 | CRITICAL | Automatico | Da fare |
| CSM-06 | Step 2: temperature monitoring | tempTimer fires, reads sim_getTemperature(). Heater toggled based on setpoint ± tolerance | CRITICAL | Automatico | Da fare |
| CSM-07 | Step 2: temperature reached | Temperature within tolerance. tempTimer deleted, transition to step 3 | CRITICAL | Automatico | Da fare |
| CSM-08 | Step 2: temperature timeout | tempTimeoutCounter exceeds limit. Timeout triggers, tempTimer deleted, process continues to step 3 | HIGH | Automatico | Da fare |
| CSM-09 | Step 3: rotation/tank check | Step 3 verifies tank presence and rotation setup. Skip available | HIGH | Automatico | Da fare |
| CSM-10 | Step 3 → 4: begin processing | Start processing. processTimer created with 1000ms interval. isDeveloping=true | CRITICAL | Automatico | Da fare |
| CSM-11 | Step 4: processTimer increments elapsed | processTimer fires every second. Elapsed time incremented. Percentage calculated via calculatePercentage() | CRITICAL | Automatico | Da fare |
| CSM-12 | Step 4: step complete → next step | Elapsed time reaches step duration. Advance to next step in list (ckup->currentStep = currentStep->next) | CRITICAL | Automatico | Da fare |
| CSM-13 | Step 4: last step complete | All steps completed. processTimer deleted, isDeveloping=false, completion UI shown | CRITICAL | Automatico | Da fare |
| CSM-14 | Step 4: multiple steps in sequence | Process with 5+ steps. Verify each step transition: elapsed resets, name updates, percentage resets | HIGH | Automatico | Da fare |

## 3.2 Stop Scenarios

8 test totali — 0 superati, 8 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| STP-01 | Stop Now during processing | Click Stop Now during step 4. Confirmation popup shown. Confirm → all timers deleted, processing halted immediately | CRITICAL | Automatico | Da fare |
| STP-02 | Stop Now – cancel | Click Stop Now, then Cancel in popup. Processing continues unchanged | HIGH | Automatico | Da fare |
| STP-03 | Stop After during processing | Click Stop After. Flag stopAfter=true set. Current step continues, stops after it ends | CRITICAL | Automatico | Da fare |
| STP-04 | Stop Now after Stop After | Click Stop After then Stop Now. handleStopNowAfterStopAfter() overrides, immediate halt | HIGH | Automatico | Da fare |
| STP-05 | Stop during water fill (step 1) | Click Stop Now during fill. pumpTimer deleted, pump relay closed, drain initiated | HIGH | Automatico | Da fare |
| STP-06 | Stop during temp wait (step 2) | Click Stop Now during temp control. tempTimer deleted, heater off, drain | HIGH | Automatico | Da fare |
| STP-07 | Close checkup after stop | After Stop Now, Close button returns to main menu. All timers NULL, no memory leak | HIGH | Automatico | Da fare |
| STP-08 | Close checkup during processing | Close button clicked while processTimer active. Timer deleted, screen closed safely | HIGH | Automatico | Da fare |

## 3.3 Timer Callbacks

5 test totali — 0 superati, 5 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| TMR-01 | processTimer callback execution | Create real lv_timer with processTimer callback. Pump LVGL. Verify elapsed incremented after 1 tick | CRITICAL | Automatico | Da fare |
| TMR-02 | tempTimerCallback execution | Create real lv_timer with tempTimerCallback. Verify temperature read and heater decision logic | HIGH | Automatico | Da fare |
| TMR-03 | pumpTimer callback execution | Create real lv_timer with pumpTimer callback. Verify fill state machine progresses | HIGH | Automatico | Da fare |
| TMR-04 | safeTimerDelete during callback | Verify that deleting a timer from within its own callback does not crash (LVGL allows this) | MEDIUM | Automatico | Da fare |
| TMR-05 | Multiple timers concurrent | processTimer + tempTimer both active. Pump LVGL. Both callbacks fire without interference | MEDIUM | Automatico | Da fare |

## 3.4 Keyboard Context

6 test totali — 0 superati, 6 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| KBD-01 | CLICKED captures context | Simulate CLICKED on process name textarea. Verify kbCtx.owner=KB_OWNER_PROCESS, kbCtx.textArea set | HIGH | Automatico | Da fare |
| KBD-02 | READY finalizes text | After typing in keyboard, READY event. Text from keyboard buffer copied to target textarea | HIGH | Automatico | Da fare |
| KBD-03 | CANCEL reverts text | Cancel keyboard. Original text preserved in target textarea | HIGH | Automatico | Da fare |
| KBD-04 | DEFOCUSED clears context | Keyboard defocus. kbCtx cleared via kb_ctx_clear() | MEDIUM | Automatico | Da fare |
| KBD-05 | Rapid owner switch | Open filter keyboard, then immediately open process name keyboard. Context switches cleanly | MEDIUM | Automatico | Da fare |
| KBD-06 | Keyboard with NULL textArea | Call event_keyboard with NULL textArea. No crash, context remains KB_OWNER_NONE | MEDIUM | Automatico | Da fare |

## 3.5 Page Lifecycle

7 test totali — 4 superati, 3 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| LCY-01 | processDetail rapid open/close | Open processDetail 10 times in succession. No memory leak, no crash | HIGH | Automatico | Da fare |
| LCY-02 | processDetail teardown resets style | Open processDetail, close. Verify textAreaStyle.values_and_props == NULL (style reset) | HIGH | Automatico | Da fare |
| LCY-03 | stepDetail open/close within processDetail | Open processDetail → open stepDetail → close stepDetail. processDetail still valid | HIGH | Automatico | Da fare |
| LCY-04 | processDetail run → checkup transition | Click Run. processDetail styles reset, checkup screen loaded. processDetailParent deleted by initCheckup | CRITICAL | Automatico | Superato |
| LCY-05 | checkup close returns to menu | From checkup, click Close. All timers deleted, checkupParent deleted, main menu loaded | HIGH | Automatico | Superato |
| LCY-06 | Settings page open/close | Switch to Settings tab, modify values, switch back to Processes. No dangling pointers | LOW | Automatico | Superato |
| LCY-07 | Tools page popups lifecycle | Open export/import/credits popups. Close each. No popup remains after close | LOW | Automatico | Superato |

## 3.6 Swipe Gestures

6 test totali — 0 superati, 6 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| SWP-01 | Step element swipe left | Swipe left on step element. Edit/Delete buttons appear | MEDIUM | Automatico | Da fare |
| SWP-02 | Step element swipe right | Swipe right after left swipe. Buttons hide, element returns to normal | MEDIUM | Automatico | Da fare |
| SWP-03 | Process element swipe left | Swipe left on process element. Delete/Duplicate buttons appear | MEDIUM | Automatico | Da fare |
| SWP-04 | Double swipe left does not duplicate buttons | Swipe left twice on same element. Buttons appear once, no duplicate UI | MEDIUM | Automatico | Da fare |
| SWP-05 | Swipe element A then swipe element B | Swipe step A left (buttons shown). Swipe step B left. A buttons hide, B buttons appear | MEDIUM | Automatico | Da fare |
| SWP-06 | Long press on step element | Long press opens stepDetail for editing (if implemented) | LOW | Manuale | Da fare |

## 3.7 Message Popups

6 test totali — 3 superati, 3 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| POP-01 | messagePopupCreate basic | Create popup with title, text, 2 buttons. All elements visible, correct text | MEDIUM | Automatico | Da fare |
| POP-02 | Stop Now confirmation popup | From checkup, click Stop Now. Popup appears with Confirm/Cancel. Confirm halts | HIGH | Automatico | Da fare |
| POP-03 | Delete process confirmation | Swipe process left, click Delete. Popup appears. Confirm removes process from list | HIGH | Automatico | Superato |
| POP-04 | Delete step confirmation | Swipe step left, click Delete. Popup appears. Confirm removes step | HIGH | Automatico | Superato |
| POP-05 | Cancel popup preserves state | Open delete popup, click Cancel. Item still in list, UI unchanged | MEDIUM | Automatico | Superato |
| POP-06 | Popup close button (X) | Click X on popup. Popup closes without executing action | LOW | Automatico | Da fare |

## 3.8 Boundary Conditions

8 test totali — 1 superati, 7 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| BND-01 | Max 30 steps per process | Add 30 steps to a process. Attempt 31st. Verify 31st is rejected or button disabled | MEDIUM | Automatico | Da fare |
| BND-02 | Max 50 processes | Add 50 processes. Attempt 51st. Verify rejection/button disabled | MEDIUM | Automatico | Da fare |
| BND-03 | Process name at MAX_PROC_NAME_LEN | Enter 20-char name. Verify stored correctly, not truncated. 21st char rejected | LOW | Automatico | Superato |
| BND-04 | Step name at max length | Enter maximum length step name. Verify stored and displayed correctly | LOW | Automatico | Da fare |
| BND-05 | Zero-length step (0:00) | Create step with 0 mins, 0 secs. Included in total? Process still runnable? | MEDIUM | Automatico | Da fare |
| BND-06 | Very long step (99:59) | Create step with 99 mins, 59 secs. Verify display, total time overflow handling | LOW | Automatico | Da fare |
| BND-07 | Empty process (0 steps) Run attempt | Try Run on process with 0 steps. Button should be disabled or error shown | MEDIUM | Automatico | Da fare |
| BND-08 | Process with 30 steps in checkup | Run process with 30 steps. Verify all steps execute, UI scrolls/paginates | LOW | Manuale | Da fare |

## 3.9 Configurazione

6 test totali — 3 superati, 3 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| CFG-01 | Config save/load round-trip | Save all processes, reload. All data identical (names, steps, temps, types) | HIGH | Automatico | Superato |
| CFG-02 | Missing config file uses defaults | Load with no config file. No crash, default values applied | HIGH | Automatico | Superato |
| CFG-03 | Settings persist across restart | Modify settings (sliders, switches). Save, reload. All values preserved | HIGH | Automatico | Superato |
| CFG-04 | Corrupted config file | Load config with partial/corrupted data. No crash, partial data loaded or defaults used | HIGH | Automatico | Da fare |
| CFG-05 | Write failure (disk full) | Simulate write failure. Error handled, no data corruption on existing file | MEDIUM | Manuale | Da fare |
| CFG-06 | Config with 50 processes + 30 steps each | Stress test: write/read 50 processes each with 30 steps. Performance and correctness | MEDIUM | Automatico | Da fare |

## 3.10 Temperatura

5 test totali — 1 superati, 4 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| TMP-01 | sim_getTemperature default | Without heater, sim_getTemperature returns 20.0f (ambient) | LOW | Automatico | Da fare |
| TMP-02 | sim_setHeater(true) raises temp | Enable heater, poll temperature over time. Verify temperature increases | MEDIUM | Automatico | Da fare |
| TMP-03 | sim_resetTemperatures returns to ambient | Heat up, then reset. Temperature returns to 20.0f | LOW | Automatico | Da fare |
| TMP-04 | Temperature tolerance check | Setpoint 38°C, tolerance 0.3. Verify acceptance at 37.8, 38.2. Rejection at 37.5, 38.5 | MEDIUM | Automatico | Da fare |
| TMP-05 | C→F conversion in UI | Switch tempUnit to Fahrenheit. Verify all temperature displays update (process detail, checkup) | LOW | Automatico | Superato |

## 3.11 Form Validation

6 test totali — 1 superati, 5 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| FRM-01 | Save process without name | Leave name empty, add steps, click Save. Save button stays disabled | MEDIUM | Automatico | Da fare |
| FRM-02 | Save process without steps | Enter name, no steps. Save button disabled or save rejected | MEDIUM | Automatico | Superato |
| FRM-03 | Step save without name | Open step detail, set time but no name. Verify behavior | MEDIUM | Automatico | Da fare |
| FRM-04 | Temperature field bounds | Enter temp 0, 100, negative. Verify bounds enforced | LOW | Automatico | Da fare |
| FRM-05 | Temp control switch disables fields | Toggle temp control off. Temp and tolerance fields become disabled | LOW | Automatico | Da fare |
| FRM-06 | Film type mutual exclusion | Select Color, then B&W. Only one active at a time | LOW | Automatico | Da fare |

## 3.12 Deep Copy / Clone

8 test totali — 8 superati, 0 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| CPY-01 | Deep copy step preserves all fields | Copy step. All 8 sStepData fields match. UI pointers NULL in copy | HIGH | Automatico | Superato |
| CPY-02 | Deep copy process preserves all fields | Copy process. All sProcessData fields match. Step list fully cloned | HIGH | Automatico | Superato |
| CPY-03 | Deep copy isolation – step | Modify copy. Original unchanged | HIGH | Automatico | Superato |
| CPY-04 | Deep copy isolation – process | Modify copy temp. Original temp unchanged | HIGH | Automatico | Superato |
| CPY-05 | Clone-in-place step | single_step_clone(src, &dst). All data fields match, different pointer | HIGH | Automatico | Superato |
| CPY-06 | Clone-in-place process | single_process_clone(src, &dst). Data + step count match | HIGH | Automatico | Superato |
| CPY-07 | Clone NULL safety | single_step_clone(NULL, dst) returns false. single_process_clone(src, NULL) returns false | MEDIUM | Automatico | Superato |
| CPY-08 | Deep copy parentProcess zeroed | Original has parentProcess set. Copy has parentProcess=NULL (not shared) | MEDIUM | Automatico | Superato |

## 3.13 Destroy / Memoria

6 test totali — 6 superati, 0 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| MEM-01 | All destroy(NULL) safe | step_detail_destroy(NULL), checkup_destroy(NULL), etc. No crash | HIGH | Automatico | Superato |
| MEM-02 | process_node_destroy cascading | Create process with steps + checkup. Destroy. All sub-objects freed | HIGH | Automatico | Superato |
| MEM-03 | emptyList step list | Create 2-step list. emptyList(). size=0, start=NULL, end=NULL | MEDIUM | Automatico | Superato |
| MEM-04 | emptyList process list | Create 2-process list. emptyList(). size=0, start=NULL, end=NULL | MEDIUM | Automatico | Superato |
| MEM-05 | safeTimerDelete NULL safety | safeTimerDelete(NULL) and safeTimerDelete(&NULL) do not crash | MEDIUM | Automatico | Superato |
| MEM-06 | allocateAndInitializeNode invalid type | allocateAndInitializeNode(99) returns NULL | LOW | Automatico | Superato |

## 3.14 Filtri

6 test totali — 5 superati, 1 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| FLT-01 | Filter by name (partial match) | Filter 'Test'. Only processes containing 'Test' shown | MEDIUM | Automatico | Superato |
| FLT-02 | Filter by B&W only | Enable B&W filter. Only B&W processes shown | MEDIUM | Automatico | Superato |
| FLT-03 | Filter preferred only | Enable preferred filter. Only preferred processes shown | MEDIUM | Automatico | Superato |
| FLT-04 | Filter both Color + B&W | Both film types selected. All processes shown | LOW | Automatico | Superato |
| FLT-05 | Filter reset | Apply filter, then reset. All processes visible | MEDIUM | Automatico | Superato |
| FLT-06 | Combined filter (name + type + preferred) | Apply all three filters simultaneously. Verify AND logic | MEDIUM | Automatico | Da fare |

## 3.15 Navigazione

4 test totali — 3 superati, 1 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| NAV-01 | Home → Menu (Start button) | Click Start. Main menu loads with 3 tabs | LOW | Automatico | Superato |
| NAV-02 | Tab Processes → Settings → Tools | Switch all three tabs in sequence. Each loads correctly | LOW | Automatico | Superato |
| NAV-03 | Rapid tab switching 20x | Switch tabs rapidly 20 times. No crash, no UI glitch, correct tab always selected | MEDIUM | Automatico | Da fare |
| NAV-04 | processDetail → stepDetail → back → back | Full navigation depth. Process → Step detail → close → close to menu | MEDIUM | Automatico | Superato |

## 3.16 Hardware (Manuale)

8 test totali — 0 superati, 8 da fare

| ID | Test | Descrizione | Priorità | Tipo | Stato |
| --- | --- | --- | --- | --- | --- |
| HW-01 | Relay activation sequence | During checkup fill, verify relay pins toggled correctly via oscilloscope/multimeter | HIGH | Manuale | Da fare |
| HW-02 | Motor speed PWM | Set rotation speed. Verify PWM duty cycle matches expected value | MEDIUM | Manuale | Da fare |
| HW-03 | Temperature sensor reading | Place sensor in known temp water. Verify displayed value matches ±1°C | HIGH | Manuale | Da fare |
| HW-04 | Heater on/off cycle | During temp control, verify heater relay toggles at setpoint ± tolerance | HIGH | Manuale | Da fare |
| HW-05 | SD card read/write | Save config, power cycle, reload. All data preserved | HIGH | Manuale | Da fare |
| HW-06 | Display 480x320 rendering | Verify all screens render correctly on physical ILI9488 display | MEDIUM | Manuale | Da fare |
| HW-07 | Touch calibration | Verify touch accuracy on capacitive panel at screen corners and center | MEDIUM | Manuale | Da fare |
| HW-08 | Full process end-to-end | Run complete C-41 process on real machine. Verify fill, heat, develop, drain, next step | CRITICAL | Manuale | Da fare |

# 4. Riepilogo per Priorità

| Priorità | Totali | Superati | Da fare | Automatici | Manuali |
| --- | --- | --- | --- | --- | --- |
| CRITICAL | 16 | 1 | 15 | 15 | 1 |
| HIGH | 37 | 14 | 23 | 33 | 4 |
| MEDIUM | 38 | 12 | 26 | 34 | 4 |
| LOW | 18 | 8 | 10 | 16 | 2 |
| TOTALE | 109 | 35 | 74 | 98 | 11 |

# 5. Roadmap Consigliata

Fase 1 (immediata): Implementare i test CRITICAL della macchina a stati del checkup (CSM-01 → CSM-14) e gli scenari di Stop (STP-01 → STP-08). Questi 22 test coprono il cuore dell'esecuzione automatica.

Fase 2 (breve termine): Timer callbacks (TMR-01 → TMR-05) e keyboard context (KBD-01 → KBD-06). Aree toccate dal refactoring recente con rischio regressione.

Fase 3 (medio termine): Page lifecycle (LCY-01 → LCY-03), boundary conditions (BND-01 → BND-08), form validation (FRM-01 → FRM-06), e filtri combinati (FLT-06).

Fase 4 (pre-rilascio): Test manuali hardware (HW-01 → HW-08) da eseguire sull'hardware reale ESP32-S3 con display ILI9488.

# 6. Note Tecniche

Tutti i test automatici usano il framework test_runner.h con macro TEST_BEGIN/TEST_END/TEST_ASSERT. I timer LVGL possono essere testati creando lv_timer reali e chiamando test_pump() per far avanzare il loop LVGL. Le funzioni statiche (processTimer, handleFirstStep, ecc.) richiedono test indiretti attraverso l'interfaccia pubblica checkup()/event_checkup().

Per i test dei timer, il pattern consigliato è: creare un processNode completo con checkup inizializzato, chiamare checkup(pn) per avviare il flusso, poi usare test_pump(N) per simulare il passaggio del tempo e verificare lo stato dopo ogni pump.